18.11.010 Purpose of residential zone districts.

The purpose of the residential zones is to provide for areas of residential development at various specified densities throughout the county. The residential zones are intended to accommodate a variety of housing types and to encourage the provision of housing for all citizens of Madera County. It is also intended that the residential zones reflect the desired development patterns in existing neighborhoods while accommodating the need for future growth.

(Ord. No. 525VV, § 3 (Exh. C), 7-18-17).

18.11.090 Purpose of the RM (residential mountain family) zone.

Purpose of the RM (residential mountain family) zone is to provide for dwelling units on varied size lots with some accessory agricultural uses. Minimum lot size of one acre. The RM zone district is intended to be applied to areas that are rural in character and is less restrictive.

(Ord. No. 525VV, § 3 (Exh. C), 7-18-17).

18.11.100 Use regulations of residential zones.

The regulations of this section apply in the residential zones unless otherwise specifically provided by footnotes indicated in the table. The uses permitted in any zone may be further limited if environmentally sensitive lands are present, pursuant to Title 16.

A.	
      Within the residential zones, no structure or improvement, or portion thereof, shall be constructed, established, or altered, nor shall any premises be used or maintained except for one or more of the purposes or activities listed in the table. It is unlawful to establish, maintain, or use any premises for any purpose or activity not listed in this section.

B.	
      All uses or activities permitted in the residential zones shall be conducted entirely within an enclosed building unless the use or activity is traditionally conducted outdoors.

C.	
      The rental of an entire single family dwelling, regardless of the tenure or length of tenancy is a permitted use in all residential zone districts.

D.	
      For any use that cannot be readily classified, the zoning administrator shall determine the appropriate use category and use subcategory pursuant to Section 18.104.010.

Use Regulations Table of Residential Zones

The uses allowed in the residential zones are shown in the attached table.

| Symbol | Description of Symbol |
| --- | --- |
| P | Use or use category is permitted. Regulations pertaining to a specific use may be referenced. |
| Z | Use is permitted with limitations, which may include location limitations or the requirement for a use or Zoning Permit. |
| C | Conditional Use Permit Required. |
| • | Use or use category is not permitted. |

Permitted Land Uses in Residential Zones

| Use Categories/Subcategories |  |  |  |  | Zones (Title 18) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P | Permitted |  | [Former] Chapter |  | 12 |  | 13 |  | 14 |  | 16 |  | 18 |  | 20 |  | 22 |  | 23 |  | 24 | 26 | 66 |
| Z | Zoning Permit |  | Zone |  | RUS |  | RX** |  | RRS |  |  |  |  |  |  |  | RMS |  | RT** |  | RUM | RRM | RM |
| C |  | Conditional Use Permit |  |  |  |  |  |  | 1 |  | 2, 2½, 3 |  | 5 |  |  | 10 |  |  |  |  |  |  |  |
| Accessory dwelling unit (less than 1 acre) |  |  |  |  |  | • |  | • |  | • |  | • |  | • |  | • | • |  |  | • | Z | • | • |
| Accessory dwelling unit or mobile home (subject to parcel size requirements and development standards as per Section 18.04.153) |  |  |  |  |  | P |  | • |  | P |  | P |  | P |  | P | P |  |  | • | P | P | P |
| Accessory living structure (as per Section 18.04.015) |  |  |  |  |  | P |  | P |  | P |  | P |  | P |  | P | P |  |  | P | P | P | P |
| Additional houses (ratio of one SFD per 40 acres) |  |  |  |  |  | • |  | • |  | • |  | • |  | • |  | • | • |  |  | • | • | • | C |
| Agricultural |  |  |  |  |  | • |  | • |  | • |  | • |  | • |  | • | P |  |  | • | • | • | P |
| Agriculturally oriented services |  |  |  |  |  | • |  | • |  | • |  | • |  | • |  | • | • |  |  | • | • | • | C |
| Airstrips |  |  |  |  |  | • |  | • |  | • |  | • |  | • |  | • | • |  |  | • | • | • | C |
| All existing dwellings in this district shall be regularly permitted uses and not considered as nonconforming uses |  |  |  |  |  | • |  | • |  | • |  | • |  | • |  | • | • |  |  | • | • | • | P |
| Animal hospitals, kennels and laboratories using animals or animal products |  |  |  |  |  | • |  | • |  | • |  | • |  | • |  | • | • |  |  | • | • | • | C |
| Bed and breakfast establishment: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1—3 guest rooms |  |  |  |  |  | • |  | • |  | C |  | C |  | C |  | C | C |  |  | • | • | • | C |
| 1—5 guest rooms |  |  |  |  |  | • |  | • |  | • |  | • |  | • |  | • | • |  |  | • | C | C | • |
| Cemeteries, Mausoleums, Crematories |  |  |  |  |  | C |  | • |  | C |  | C |  | C |  | C | C |  |  | • | C | C | C |
| Churches and Places of Religious Assembly |  |  |  |  |  | C |  | • |  | C |  | C |  | C |  | C | C |  |  | C | C | C | C |
| Forestry |  |  |  |  |  | • |  | • |  | • |  | • |  | • |  | • | P |  |  | • | • | • | P |
| Guest house (no kitchen) (as per Section 18.04.235) |  |  |  |  |  | • |  | • |  | P |  | P |  | P |  | P | P |  |  | • | P | P | P |
| Laboratories for testing, experimental or analytical purposes |  |  |  |  |  | • |  | • |  | • |  | • |  | • |  | • | • |  |  | • | • | • | C |
| Mobilehome parks, travel trailer park or tent camp |  |  |  |  |  | C |  | • |  | C |  | C |  | C |  | C | C |  |  | • | • | C | C |
| Multiple family dwelling |  |  |  |  |  | • |  | • |  | • |  | • |  | • |  | • | • |  |  | C | P | P | • |
| Non-commercial raising of animals (1)(2)(3) |  |  |  |  |  | • |  | • |  | • |  | P |  | P |  | P | • |  |  | • | • | • | P |
| Private clubs and outdoor recreational facilities |  |  |  |  |  | • |  | • |  | • |  | • |  | • |  | • | • |  |  | • | • | • | C |
| Private stables |  |  |  |  |  | • |  | • |  | • |  | P |  | P |  | P | P |  |  | • | • | • | P |
| Professional office |  |  |  |  |  | • |  | • |  | • |  | • |  | • |  | • | • |  |  | • | C | C | • |
| Public and commercial refuse disposal sites |  |  |  |  |  | • |  | • |  | • |  | • |  | • |  | • | • |  |  | • | • | • | C |
| Public stables |  |  |  |  |  | • |  | • |  | • |  | • |  | • |  | • | C |  |  | • | • | • | C |
| Residential care facilities: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 or fewer persons |  |  |  |  |  | P |  | P |  | P |  | P |  | P |  | P | P |  |  | P | P | P | P |
| 7 or more persons |  |  |  |  |  | C |  | C |  | C |  | C |  | C |  | C | C |  |  | C | C | C | C |
| Residential conversion |  |  |  |  |  | C |  | C |  | • |  | • |  | • |  | • | • |  |  | • | • | • | • |
| Single family dwelling or mobile home |  |  |  |  |  | P |  | P |  | P |  | P |  | P |  | P | P |  |  | P | P | P | P |

1.	
      The noncommercial raising of bovine animals, equine animals, swine, sheep and goats where the lot area is one acre or more provided that the number thereof in any combination shall not exceed four adult animals per acre, with not more than two adult animals and three immature offspring of a bovine, equine type, or combination thereof per acre.

2.	
      The noncommercial raising of poultry, rabbits and other similar small animals where the lot area is one acre or more provided that the number per acre, or combination thereof, shall not exceed twenty-four.

3.	
      All pens, coops, stables or barns for the keeping of the above indicated animals must comply with the structure setback requirements for this district and shall not be constructed within fifty feet of any structure designed for human habitation.

** This zone district would only be allowed to be used in conjunction with the village core overlay district.

(Ord. No. 525VV, § 3 (Exh. C), 7-18-17).

18.11.110 Development regulations of residential zones.

A.	
      Within the residential zones, no structure or improvement shall be constructed, established, or altered, nor shall any premises be used unless the premises complies with the regulations and standards in this division and with any applicable development regulations.

B.	
      The regulations in this section apply to all development in the residential zones whether or not a permit or other approval is required except where specifically identified.

C.	
      Temporary uses may be permitted in the residential zones for a limited period of time with a zoning permit in accordance with Chapter 18.104.

D.	
      For any use that cannot be readily classified, the zoning administrator shall determine the appropriate use category and use subcategory pursuant to Chapter 18.104.

(Ord. No. 525VV, § 3 (Exh. C), 7-18-17).

18.11.120 Development standards for residential zones.

The development standards table for residential zones identifies the parcel size, density standards, lot coverage, setbacks and height regulations that apply in the residential zones.

Parcel Size, Density and Setback for Residential Zones

| Use Categories/Subcategories |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Former] Chapter | 12 | 13 | 14 | 16 | 18 | 20 | 22 |  | 23 |  | 24 | 26 | 66 |
| Zone | RUS | RX** | RRS |  |  |  | RMS |  | RT** |  | RUM | RRM | RM |
|  |  |  | 1 | 2, 2½, 3 | 5 | 10 |  |  |  |  |  |  |  |
| Maximum permitted density (DU per lot) | 2 |  | 1 | 2 | 2 | 2 | 2 |  | 1 |  | 2 | 2 | 2* |
| Min. lot area | 4500 | 3000 | 1 ac | 2, 2.25, 2.7 | 4.5 | 9 | 1 |  | 2200 2000/family |  | 6000 2000/family | • | 2 |
| Min. lot dimensions: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lot width (ft.), average | 50 | 35 | 120 | 200 250 250 | 270 | 400 | • |  | 25 |  | 65 | • | 120 |
| Lot depth (ft.), minimum | • | 50 | • | • | • | • | • |  | 80 |  | • | • | • |
| Setback requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Front setback (ft.) from right-of-way | 25 | 15 | 25 | 25 | 25 | 25 | 25 |  | 5 |  | 25 | 25 | 25 |
| Side setback (ft.) — over 1 ac. under 1 ac. | 3.5 | 5 | 10 | 10 | 10 | 10 | 10 5/10 |  | 0.5 |  | 5 | 25(M) 10(S) | 10 |
| Rear setback (ft.) | 10 | 10 | 20 | 20 | 20 | 20 | 20 |  | 3 |  | 10 | 20 | 20 |
| Max. structure height | 35 | 35 | 35 | 35 | 35 | 35 | 35 |  | 40 |  | 40 | 40 | 40 |
| Accessory building height | 15 | 15 | 15 | 15 | 15 | 15 | 15 |  | • |  | 15 | 15 | 15 |
| Agricultural building height | • | • | • | • | • | • | 60 |  | • |  | • | • | 60 |
| Length to width ratio | 3:1 | • | 3:1 | 3:1 | 3:1 | 3:1 | 3:1 |  | • |  | 3:1 | 3:1 | 3:1 |
| Max. lot coverage (%) | 80 | 80 | 40 | 20 | 20 | 20 | • |  | 75 |  | 80 | 35 | 40 |
| Max. floor area ratio (%) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 story building | 60 | 80 | 10 | 5 | 5 | 5 | • |  | 75 |  | 80 | 20 | 10 |
| 2 story building | 60 | 80 | 10 | 5 | 5 | 5 | • |  | 110 |  | 80 | 20 | 10 |
| 3 story building | 60 | 80 | 10 | 5 | 5 | 5 | • |  | 150 |  | 80 | 20 | 10 |
| Parking spaces required | 2 | 2 | 2 | 2 | 2 | 2 | 2 |  | 2 |  | 2 | 2 | 2 |

*Additional houses, not to exceed a ratio of one dwelling unit per forty acres in the ranch or farm.

**Only allows in the village core overlay district.

Setbacks are measured from the property line to the wall of the structure. Front setbacks are measured from the road right-of-way to the wall of the structure.

If two or more roadways border the parcel, the front of the house will require front (twenty-five feet from road right-of-way) setbacks. If a driveway accesses a different roadway from the front of the house, a twenty-five-foot setback from the road right-of-way will be required.

(Ord. No. 525VV, § 3 (Exh. C), 7-18-17).